UHFGate

User Manual V1.0

Contents

Revision history	4

1 Interface specification	4

2 Protocol Description	4

3 Formats of data frames	5

3.1 Command Frame Format (From HOST)	5

3.2 Response Frame Format (From Controller)	5

3.3 CRC16	6

4 List of the response status	8

5 Command detail description	9

5.1 Get Controller Information	9

5.2 Set controller configuration command	9

5.3 Get controller configuration command	9

5.4 Get Message	10

5.5 ACK	12

5.6 Relay	12

5.7 Buzzer Controll	13

5.8 LED Controll	13

5.9 Set Clock	14

5.10 Get Clock	14

5.11 Clear Message	14

5.12 Get Statistics	15

5.13 Set Statistics	15

5.14 Set GPIO	15

5.15 Get GPIO	16

5.16 Connection status with reader	16

5.17 Restart	16

5.18 Restore default	17

5.19 Antenna return loss	17

6 Configuration parameter	17

6.1 Controller configuration parameters table	18

6.2.1 CFG1: Address	20

6.2.2 CFG2: Work Mode	20

6.2.3 CFG3: Alarm Mode	20

6.2.4 CFG4: Event Enable	21

6.2.5 CFG5: Custom Alarm Rule	21

6.2.6 CFG6: Alarm Event Action	21

6.2.7 CFG7: Message	22

6.2.8 CFG8: Message Transmission	23

6.2.9 CFG9: Person direction	25

6.2.10 CFG10: Forward Event	25

6.2.11 CFG11: Backward Event	26

6.2.12 CFG12: IR Trigger Enable	26

6.2.13 CFG13: Tag filter Time	27

6.2.14 CFG30: Inventory Type	27

6.2.15 CFG31: Inventory Parameter	27

6.2.16 CFG32: TID Parameter	28

6.2.17 CFG33: Mask Set	28

6.2.18 CFG34: Work Region	28

6.2.19 CFG35: RF Power	29

6.2.20 CFG36: Antenna	29

6.2.21 CFG50: IP Address	29

6.2.22 CFG51: Connection	30

6.2.23 CFG52: Repeat Connect	30

6.2.24 CFG53: HeartBeat And Keepalive	30

7．Network configuration	32

7.1 Network Parameter config	32

7.2 Search Controller IP	32

- Revision history

| Date | Version | Content |
| --- | --- | --- |
| 2019-7-17 | V1.0 | First |

- 1 Interface specification

Communication between host and controller is based on RS232or TCP/IP. RS232 in internal extends all the way RS485, so RS232 and RS485 and can only use one of the interfaces.

| Data Frame Format | 1 start bit, 8 data bits, 1 stop bit |
| --- | --- |
| Parity | NONE |
| Default Baud Rate | 38400 bps |
| Transmission order | Least-significant bit first |

TCP/IP default work as TCP server，local port is 6000，static IP: 192.168.1.192.

- 2 Protocol Description

In a typical communication, host delivers command and parameters to controller, controller should execute the command and provide result or data response. controller will receive and execute ONE command in every communication cycle

Command delivery from host:

| Host | Direction | Controller |
| --- | --- | --- |
| Command frame | → |  |

Once a command is received by controller, the controller will produce a response to host within the predefined inventory time. Data communication time is negligible here.

During communication, the transmitting time gap between every two consecutive bytes should be less than 15ms. If the time gap exceeds 15ms, all the received data should be discarded and a normal receiving process restarts from the next byte.

Response delivery from reader:

| Controller | Direction | Host |
| --- | --- | --- |
| Response frame | → |  |

Description:

During communication, the transmitting time gap between every two consecutive bytes should be less than 15ms.

A typical communication process is stated below:

1. Host delivers command to Controller and wait for response;

2. Controller receives command, starts specific operation according to the received frame and produce response (of operation status and/or data) to host;

3. Host receives response from Controller.

- 3 Formats of data frames

- 3.1 Command Frame Format (From HOST)

| Len | Addr | Cmd | Data[] | LSB-CRC16 | MSB-CRC16 |
| --- | --- | --- | --- | --- | --- |

Format definition:

|  | Length (in byte) | Description |
| --- | --- | --- |
| Len | 1 | Length of the command frame, defined by the amount of bytes between Adr and MSB_CRC16, i.e. the length of Data[] + 4. The minimum and maximum values of Len are 4 and 255. |
| Adr | 1 | Reader device address, the valid range of device address is 0x00 ~ 0xFE. 0xFF is the broadcast address. Reader will only response to command frame stated with a identical address or 0xFF. Default device address: 0x00. |
| Cmd | 1 | Command code. |
| Data[] | Variable | Parameter field, host may or may not provides parameters according to different commands formats. |
| LSB-CRC16 | 1 | Low byte of CRC16. These 2 bytes are the CRC16 calculation of Len to Data[]. |
| MSB-CRC16 | 1 | High byte of CRC16 |

Note: when the command block does not conform to the requirements, the controller will not have any response

- 3.2 Response Frame Format (From Controller)

| Len | Adr | reCmd | Status | Data[] | LSB-CRC16 | MSB-CRC16 |
| --- | --- | --- | --- | --- | --- | --- |

Format description：

|  | Length (in byte) | Description |
| --- | --- | --- |
| Len | 1 | Length of the response frame, defined by the amount of bytes between Adr and MSB_CRC16), i.e. the length of Data[] + 5. |
| Adr | 1 | Reader address. |
| reCmd | 1 | reCmd indicates which command this frame is responding to. For unrecognised command, reCmd should be 0x00. |
| Status | 1 | Command execution status/result |
| Data[] | Variable | Data field, reader may or may not provide parameters according to actual commands. |
| LSB-CRC16 | 1 | Low byte of CRC16. These 2 bytes are the CRC16 calculation of Len to Data[].。 |

- 3.3 CRC16

C language algorithm of CRC16:

uint16 crc16(unsigned char *pData, int len) { const int16 POLYNOMIAL=0x8408; const int16 PRESET_VALUE=0xffff; int i,j; unsigned int current_crc_value=PRESET_VALUE; for(i=0;i<len;i++) /*len=number of protocol bytes without CRC*/ { current_crc_value=current_crc_value^((unsigned int)pData[i]); for(j=0;j<8;j++) { if(current_crc_value&0x0001) { current_crc_value=(current_crc_value>>1)^POLYNOMIAL; } else { current_crc_value=(current_crc_value>>1); } } } return current_crc_value; }

pucY is the pointer of the array entry for CRC16 calculation, ucX is the amount of bytes included in this calculation. A 0x0000 return value indicates the input data had passed the CRC16 verification.

Whenever the host receive a data frame from reader, the host should always verify the frame by calling this CRC16 algorithm.

- 4 List of the response status

| Status | Status definition | Description |
| --- | --- | --- |
| 0x00 | Operation succeed | Command is successfully executed, all the requested data is included in the Data[] field of response frame. |
| 0x01 | Command length error | Host delivered a command frame with incorrect frame length |
| 0x02 | Illegal command | Host delivered a incorrect command frame, e.g. unrecognised command code or a command frame with CRC error (failed to pass CRC16 check) |
| 0x03 | Parameter error | Host delivered unrecognised parameters in a command frame. |
| 0x06 | FlashWrite Error | device configuration command returns the error code when save parameters failed |
| 0x07 | Command operation error | Command execution location error, returns the error code. |

- 5 Command detail description

- 5.1 Get Controller Information

Use this command to get controller system version information.

Command frame:

| Len | Addr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x04 | 0xXX | 0x21 | —— | LSB | MSB |

Response frame:

| Len | Addr | Cmd | Status | Data[] |  |  | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x0c | 0xXX | 0x21 | 0x00 | Type | Version | DevSN | LSB | MSB |
|  |  |  |  | 1byte | 2bytes | 4bytes |  |  |

Parameters definition

Type：1 byte, controller code.

Version：2bytes, controller Firmware version, the high byte states the main version number and the low byte states the subversion number.

DevSN：4bytes, Controller serial number.

- 5.2 Set controller configuration command

Used to configure the parameters of the controller

Command frame:

| Len | Addr | Cmd | Data[] |  | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05+n | 0xXX | 0x22 | CFGNo | CFGData (n bytes) | LSB | MSB |

Command data block contains CFGNo used to specify configuration sequence number, CFGData is to configure the parameters of the content, see

Configuration parameter

- 5.3 Get controller configuration command

Used to get the parameters of the controller

Command frame:

| Len | Addr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | CFGNo |  |  |
| 0x05 | 0xXX | 0x23 | 0xXX | LSB | MSB |

Response frame:

| Len | Addr | Cmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05+n | 0xXX | 0x23 | 0x00 | CFGData) | LSB | MSB |
|  |  |  |  | nbytes |  |  |

Command data block contains CFGNo used to specify configuration sequence number, CFGData is to configure the parameters of the content, see

Configuration parameter

- 5.4 Get Message

Used to get message about tag and person.

Command frame:

| Len | Addr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x04 | 0xXX | 0x24 | —— | LSB | MSB |

Response frame:

| Len | Addr | Cmd | Status | Data[] |  |  | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0x24 | 0x00 | PacketNo | MsgType | Data | LSB | MSB |
|  |  |  |  | 0xXX | 0xXX | n Byte |  |  |

PacketNo:1 byte, Message number. Each send a message, the message sequence number automatic one, beyond the scope of a byte will back to 0 to accumulate. Used to distinguish between new messages and repeat.

MsgType:1 byte,Message Type：

0x00：null message,Data[] is null.

0x01: Inventory Message.

0x02: EAS Alarm Message

0x03: Person Message.

0x04: IR Message

Data：n Byte.the format on which type as follows:

(1) Inventory Message.

| Data[] |  |  |
| --- | --- | --- |
| Time | Num |  |
| 6bytes | 1byte | EPC-1, EPC-2, EPC-3… |

Time：Message produce time,Each byte followed by days, hours, minutes, seconds, milliseconds (high byte), ms (low byte). Days, hours, minutes and seconds the system 24 hours a day. Milliseconds for two bytes, the range of 0 ~ 999, the high byte in the former

Num:the number of tag in this message

EPC ID: inquired EPC/TID data. For instance, EPC-1 is the EPC/TID length + EPC number/TID data of the first tag etc. The EPC length values are one byte parameters. The unit of EPC C1 G2 data is word. During communication, the most-significant word and the most-significant byte in a word will be transmitted first.

(2) EAS Alarm Message

| Data[] |  |  |
| --- | --- | --- |
| Time | EASType | EPC ID |
| 6bytes | 1byte | n byte |

Time：Message produce time,Each byte followed by days, hours, minutes, seconds, milliseconds (high byte), ms (low byte). Days, hours, minutes and seconds the system 24 hours a day. Milliseconds for two bytes, the range of 0 ~ 999, the high byte in the former

EASType：alarm type ,0-no alarm；1-EAS Alarm(NXP)；2-Emulate EAS alarm

EPC ID：inquired EPC/TID data. For instance, EPC ID is the EPC/TID length + EPC number/TID data.

Note：EPCID Data only when Emulated EAS alarm have a return. Standard EAS alarm has null

(3) IR Message

| Data[] |  |  |
| --- | --- | --- |
| Time | Channel | Active |
| 6bytes | 1byte | 1byte |

Time：Message produce time,Each byte followed by days, hours, minutes, seconds, milliseconds (high byte), ms (low byte). Days, hours, minutes and seconds the system 24 hours a day. Milliseconds for two bytes, the range of 0 ~ 999, the high byte in the former

Channel：IR line information 。

0：line 1 first IR；

1：line 1 second IR；

2：line 2 first IR；

3：line 2 second IR；

Active: IR status，0-block；1-sync

(4) Person Message

| Data[] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Time | In/Out | positive | reverse | Online |
| 6bytes | 0xXX | 4bytes | 4bytes | 4bytes |

Time：Message produce time,Each byte followed by days, hours, minutes, seconds, milliseconds (high byte), ms (low byte). Days, hours, minutes and seconds the system 24 hours a day. Milliseconds for two bytes, the range of 0 ~ 999, the high byte in the former

In/Out：person direction for this time

0：forward

1，backward

2，indefinite.

Positive：forward number.

Reverse：backward number

Online：online number.

- 5.5 ACK

The response of the host computer receives the right message for command, sent to the confirmation of the controller. Through the configuration (CFG8: message transmission) can set the controller whether need to send this order to confirm

the response.

Command frame:

| Len | Addr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x04 | 0xXX | 0x25 | —— | LSB | MSB |

None response.

- 5.6 Relay

Used to control the relay action

Command frame:

| Len | Dev_Adr | Cmd | Data[] |  |  | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x07 | 0xXX | 0x26 | No | Status | Time | LSB | MSB |
|  |  |  | 0xXX | 0xXX | 0x00 |  |  |

No :relay number，0-relay 1;1relay 2.

Status: 0x00-release，0x01-active

Time:relay active time(Time*100ms)，when Status>0x01,relay will release when timeout,if set to 0，relay active forever.

Response frame:

| Len | Addr | Cmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x26 | 0x00 | —— | LSB | MSB |

- 5.7 Buzzer Controll

Used to control the buzzer action.

Command frame:

| Len | Addr | Cmd | Data[] |  |  |  | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | R0 |  |  |
| 0x08 | 0xXX | 0x27 | 0xXX | 0xXX | 0xXX | 0x00 | LSB | MSB |

T1：beep on time，unit 10ms。

T2：beep off time, unit 10ms

T3：active count.

R0：The volume and value of 0 to 10. 0 to mute, 10 volume is the largest.

Response frame:

| Len | Addr | Cmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x27 | 0x00 | —— | LSB | MSB |

- 5.8 LED Controll

Used to control the LED action.

Command frame:

| Len | Addr | Cmd | Data[] |  |  |  | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | LED | LSB | MSB |
| 0x08 | 0xXX | 0x28 | 0xXX | 0xXX | 0xXX | 0xXX |  |  |

T1：LED on time，unit is 100ms

T2：LED off time，unit is 100ms

T3：LED flash count.

LED：flash LED color

Bit0 channel 1 red

Bit1 channel 1 blue

Bit2 channel 2 red

Bit3 channel 2 blue

Response frame:

| Len | Addr | Cmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x29 | 0x00 | —— | LSB | MSB |

- 5.9 Set Clock

Used to Set clock time.

Command frame:

| Len | Addr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Time |  |  |
| 0x0A | 0xXX | 0x29 | 6bytes | LSB | MSB |

Time: From left to right, each byte order for the year, month, day, hours, minutes and seconds. System time format for 24 hours

Response frame:

| Len | Addr | Cmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x29 | 0x00 | —— | LSB | MSB |

- 5.10 Get Clock

Used to get clock current time.

Command frame:

| Len | Addr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x04 | 0xXX | 0x2A | —— | LSB | MSB |

Response frame:

| Len | Addr | Cmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x0B | 0xXX | 0x2A | 0x00 | Time | LSB | MSB |

Time: From left to right, each byte order for the year, month, day, hours, minutes and seconds. System time format for 24 hours

- 5.11 Clear Message

Used to clear controller buffer message.

Command frame:

| Len | Addr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x04 | 0xXX | 0x2B | —— | LSB | MSB |

Response frame:

| Len | Addr | Cmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x2B | 0x00 | —— | LSB | MSB |

- 5.12 Get Statistics

Used to get controller statistics message about person and alarm.

Command frame:

| Len | Addr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x04 | 0xXX | 0x2C | —— | LSB | MSB |

Response frame:

| Len | Addr | Cmd | Status | Data[] |  |  |  | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ComeIn | GoOut | Online | Alarm |  |  |
| 0x15 | 0xXX | 0x2C | 0x00 | 4bytes | 4 bytes | 4 bytes | 4 bytes | LSB | MSB |

ComeIn：forward number，High byte in the front.

GoOut：backward number，High byte in the front.

Online：online number，High byte in the front.

Alarm：Alarm number，High byte in the front.

- 5.13 Set Statistics

Used to get controller statistics message about person and alarm.

Command frame:

| Len | Addr | Cmd | Data[] |  |  |  | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ComeIn | GoOut | Online | Alarm |  |  |
| 0x14 | 0xXX | 0x2D | 4bytes | 4bytes | 4bytes | 4bytes | LSB | MSB |

ComeIn：forward number，High byte in the front.

GoOut：backward number，High byte in the front.

Online：online number，High byte in the front.

Alarm：Alarm number，High byte in the front.

Response frame:

| Len | Addr | Cmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x2D | 0x00 | —— | LSB | MSB |

- 5.14 Set GPIO

Used to set controller gpio status.

Command frame:

| Len | Addr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | GPIO_OUT |  |  |
| 0x05 | 0xXX | 0x2E | 0xXX | LSB | MSB |

GPIO_OUT: bit3-bit0 means Out4~Out1,0-null，1-high.

Response frame:

| Len | Addr | Cmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x2E | 0x00 | —— | LSB | MSB |

- 5.15 Get GPIO

Used to get controller gpio status.

Command frame:

| Len | Addr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x04 | 0xXX | 0x2F | —— | LSB | MSB |

Response frame:

| Len | Addr | Cmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x06 | 0xXX | 0x2F | 0x00 | GPI | LSB | MSB |
|  |  |  |  | 0xXX |  |  |

GPI：GPIO status，bit3-bit0 means input4~ input1,0-null，1-high.

- 5.16 Connection status with reader

Used to detect the connection status between controller and reader.

Command frame:

| Len | Addr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x04 | 0xXX | 0x30 | —— | LSB | MSB |

Response frame:

| Len | Addr | Cmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x06 | 0xXX | 0x30 | 0x00 | ConnectStatus | LSB | MSB |
|  |  |  |  | 0xXX |  |  |

ConnectStatus：connect status，bit0-reader1，bit4-reader2,bit value fix 0 disconnect，fix 1 connect.

- 5.17 Restart

Used to restart the controller

Command frame:

| Len | Addr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x04 | 0xXX | 0x31 | —— | LSB | MSB |

Response frame:

| Len | Addr | Cmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x31 | 0x00 | —— | LSB | MSB |

- 5.18 Restore default

Used to restore the default parameter of controller.

Command frame:

| Len | Addr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x04 | 0xXX | 0x32 | —— | LSB | MSB |

Response frame:

| Len | Addr | Cmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x32 | 0x00 | —— | LSB | MSB |

- 5.19 Antenna return loss

Used to measure the return loss of antenna.

Command frame:

| Len | Addr | Cmd | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- |
| 0x05 | 0xXX | 0x33 | Ant | LSB | MSB |
|  |  |  | 0x00 |  |  |

Ant: the selected antenna. 0-7 means antenna1~antenna8.

Response frame:

| Len | Addr | Cmd | Status | Data[] | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0x06 | 0xXX | 0x33 | 0x00 | ReturnLoss | LSB | MSB |
|  |  |  |  | 0xXX |  |  |

ReturnLoss: Antenna return loss,unit is dB.

- 6 Configuration parameter

About the controller control all configuration parameters are listed in this section, convenient user unified set and get

Get parameter used command 5.3 Get controller configuration command

Set parameter used command 5.2 Set controller configuration command

After modify configuration, don’t need restart conttoller except network parameter

- 6.1 Controller configuration parameters table

configuration table：

| No. | Name | Length (byte) | describe |
| --- | --- | --- | --- |
| 1 | Address | 1 | Controller Address |
| 2 | Work Mode | 1 | Work mode: Inventory mode ,EAS Mode |
| 3 | Alarm Mode | 1 | Alarm mode: EAS(NXP)、emulate EAS |
| 4 | Event enable | 4 | Various events enabled, such as open alarm events。 |
| 5 | Custom alarm rule | 128 | According to the Bit mask way custom alarm rules |
| 6 | Alarm event action | 16 | controller action When alarming, such as action buzzer, LED lights |
| 7 | Message | 4 | The message type, such as messages, traps of personnel entering and leaving |
| 8 | Message transmission | 2 | Message transmission and validation mechanism |
| 9 | Person direction | 1 | Define into and out of the direction |
| 10 | Person into event | 16 | the action of the controller when person forward |
| 11 | Person out even | 16 | the action of the controller when person backward |
| 12 | IR trigger enable | 2 | IR trigger function |
| 13 | tag filter time | 1 | A single tag filtering time |
| … | … |  |  |
| 30 | Inventory Type | 1 | Inventory EPC or TID |
| 31 | Inventory Param | 2 | Q and Session |
| 32 | TID Param | 2 | Set the start address and length |
| 33 | Mask set | 37 | Inventory mask set |
| 34 | RF region | 3 | Set RFID work region |
| 35 | RF Power | 8 | Set RFID work power |
| 36 | Antenna | 1 | Set effective antenna |
| … | … |  |  |
| 50 | Network IP address | 13 | Set the controller ip information |
| 51 | Network Connection | 8 | Set the TCP mode, remote IP and port information |
| 52 | Network Repeat connect | 4 | Set up the network timeout disconnect and bolt connection |
| 53 | Network Heartbeat and keepalive | 4 | set heartbeat and keepalive parameter |

6.2 Controller configuration parameters function

- 6.2.1 CFG1: Address

This configuration is defined the correspondence address of the controller.

For the controller is not native or address 0 xFF command, the controller does not respond.

| Byte | 0 |
| --- | --- |
| Contents | Addr |
| Default | 0x00 |

Addr：New Address,range is 0x00-0xFE.

- 6.2.2 CFG2: Work Mode

Controller supports two work mode, inventory mode and EAS mode.

Inventory mode used to query multiple tag EPC/TID. Will not trigger the alarm.

EAS mode used to detect alarm, get in line with the alarm tag will send alarm signal

| Byte | 0 |
| --- | --- |
| Contents | WorkMode |
| Default | 0x00 |

WorkMode：0x00 Inventory Mode

0x01 EAS Mode

- 6.2.3 CFG3: Alarm Mode

In EAS mode, you can set different alarm mode, meet different application scenarios.

| Byte | 0 | 1 |
| --- | --- | --- |
| Contents | AlarmMode | AlarmType |
| Default | 0x00 | 0x00 |

AlarmMode: 	0x00 EAS Alarm(NXP)

0x01 Emulated EAS Alarm

AlarmType：when set Emulated EAS Alarm，this parameter define alarm type.

0x00: Mode A, EPC 92,93bit value is 01alarm.

0x01: Mode B, EPC first bit is 0 alarm.

0x02: Mode C, alarm with any tag.

0x03: Mode D, Custom alarm rules. See CFG5

- 6.2.4 CFG4: Event Enable

Enable event, when enabled，actions as follows:

| Byte | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Contents | EventEn | 0x00 | 0x00 | 0x00 |
| Default | 0x01 | 0x00 | 0x00 | 0x00 |

EventEn:define event as bit,0-disable,1-enable.

Bit0- Alarm Event

Bit1- Forward Event

Bit2- Backward Event

- 6.2.5 CFG5: Custom Alarm Rule

Custom rules support Bit mask set alarm rules.

Controller working in emulated EAS alarm mode, and select the custom EAS alarm, after the success for tag number of EPC, preferred to judge whether the length of the EPC between MinLen and MaxLen, does not conform to the discarded directly, to conform to the EPC, it Bit by Bit from the top byte EPC to mask the comparison, the check corresponding MaskEn buy 1 Bit in the EPC number with corresponding Bit MaskData values are the same, different is discarded the EPC number, all the same just call the police.

| Byte | 0-61 | 62-123 | 124 | 125 | 126 | 127 |
| --- | --- | --- | --- | --- | --- | --- |
| Contents | MaskEn | MaskData | MinLen | MaxLen | 0x00 | 0x00 |
| Default | 0x00 | 0x00 | 0x00 | 0x | 0x00 | 0x00 |

MaskEn: Mask Enable bit.

MaskData: Mask data.

MinLen: Minimum EPC word number length.

MaxLen: Maximum EPC word number length.

- 6.2.6 CFG6: Alarm Event Action

Controller action When alarming, such as action buzzer, LED lights

| Byte | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contents | BuzzOpenT | BuzzcloseT | BuzzCnt | BuzzVol | LEDEn | LEDOpenT | LEDCloseT |
| Default | 0x00 | 0x00 | 0x00 | 0x00 | 0x05 | 0x0A | 0x0A |
| Byte | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| Contents | LEDCnt | Relay1T | Relay2T | BeepT | 0x00 | 0x00 | 0x00 |
| Default | 0x01 | 0x00 | 0x00 | 0x01 | 0x00 | 0x00 | 0x00 |
| Byte | 14 | 15 |  |  |  |  |  |
| Contents | 0x00 | 0x00 |  |  |  |  |  |
| Default | 0x00 | 0x00 |  |  |  |  |  |

BuzzOpenT: Buzzer on time, unit 10ms.

BuzzcloseT: Buzzer off time, unit 10ms.

BuzzCnt: Buzzer action count.

BuzzVol: The volume of the buzzer. Value of 0 to 10, 0 to silent, 10 for the volume is the largest.

LEDEn: Action LED, The specified bit fix 1 can make the corresponding LED

Bit0: Channel 1 red.

Bit1: Channel 1 blue.

Bit2: Channel 2 red.

Bit3: Channel 2 blue.

LEDOpenT: LED on time, unit 100ms

LEDcloseT: LED off time, unit 100ms

LEDCnt: LED flash Time.

Relay1T: Relay 1 active time, unit 100ms.

Relay2T: Relay 1 active time, unit 100ms.

BeepT: onboard buzzer on time,unit 10ms.

- 6.2.7 CFG7: Message

The generation of this configuration controller messages defined

| Byte | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Contents | MessageEn | 0x00 | 0x00 | 0x00 |
| Default | 0x01 | 0x00 | 0x00 | 0x00 |

MessageEn: According to the bit define which information allows to upload.

Bit0: Person message

Bit1: IR Message

Bit2: No Alarm tag Message

- 6.2.8 CFG8: Message Transmission

This configuration is defined controller message transmission and confirm the mechanism

| Byte | 0 | 1 |
| --- | --- | --- |
| Contents | MsgAutoUpload | MsgAck |
| Default | 0x00 | 0x01 |

MsgAutoUpload: News initiative to upload. 1- auto upload, 0-no active upload need to send the GetMessage command to get the message.

MsgAck: Need ACK flag, 0- no ACK, 1-need ACK.

Auto upload data format:

| Len | Addr | Cmd | Status | Data[] |  |  | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0xEE | 0x00 | DevSN | MsgType | Data | LSB | MSB |
|  |  |  |  | 4 Byte | 0xXX | n Byte |  |  |

DevSN: 4 bytes, device ID.

MsgType: Message Type,

0x00: null, Data[] is null;

0x01: Inventory Message;

0x02: EAS Alarm Message;

0x03: Person Message;

0x04: IR Message;

Data: n Byte

(1) Inventory Message

| Data[] |  |  |
| --- | --- | --- |
| Time | Num | EPC-1, |
| 6bytes | 1byte | N bytes |

Time: The message of time. From left to right, each byte followed by days, hours, minutes, seconds, milliseconds (high byte), ms (low byte). USES the system 24 hours one day. Milliseconds for two bytes, the range of 0 ~ 999, the high byte in the front.

Num: tag number.

EPC ID: The inquired tag data, the data block follows the format stated below:

EPC/TID length + EPC/TID No.

(2) EAS Alarm Message

| Data[] |  |  |
| --- | --- | --- |
| Time | EASType | EPC ID |
| 6bytes | 1byte | n bytes |

Time: The message of time. From left to right, each byte followed by days, hours, minutes, seconds, milliseconds (high byte), ms (low byte). USES the system 24 hours one day. Milliseconds for two bytes, the range of 0 ~ 999, the high byte in the front.

EASType: EAS Alarm Type.0-no EAS alarm; 1-EAS alarm(NXP); 2-Emulated EAS alarm.

EPC ID: Alarm tag EPC

Note: the data of EPCID only when Emulated EAS alarm set to have a return. Standard EAS alarm no this field

(3) Person Message

| Data[] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Time | In/Out | positive | reverse | Online |
| 6bytes | 0xXX | 4bytes | 4bytes | 4bytes |

Time: The message of time. From left to right, each byte followed by days, hours, minutes, seconds, milliseconds (high byte), ms (low byte). USES the system 24 hours one day. Milliseconds for two bytes, the range of 0 ~ 999, the high byte in the front.

In/Out: Current direction.

0: Forward

1: Backward

2: indefinite.

Positive: Forward number

Reverse: Backward number

Online: onLine number

(4)IR Message

| Data[] |  |  |
| --- | --- | --- |
| Time | Channel | Active |
| 6bytes | 1byte | 1byte |

Time: The message of time. From left to right, each byte followed by days, hours, minutes, seconds, milliseconds (high byte), ms (low byte). USES the system 24 hours one day. Milliseconds for two bytes, the range of 0 ~ 999, the high byte in the front.

Channel: the IR line

0: the line 1 of Channel 1

1: the line 2 of Channel 1

2: the line 1 of Channel 2

3: the line 2 of Channel 2

Active: IR status, 0-block; 1-Sync.

- 6.2.9 CFG9: Person direction

Define person into and out of the direction

| Byte | 0 |
| --- | --- |
| Contents | Direction |
| Default | 0x00 |

Direction: 0x00 -Forward,

0x01 -Backward

- 6.2.10 CFG10: Forward Event

Define the action of the controller when person forward

| Byte | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contents | BuzzOpenT | BuzzcloseT | BuzzCnt | BuzzVol | LEDEn | LEDOpenT | LEDCloseT |
| Default | 0x00 | 0x00 | 0x00 | 0x00 | 0x0A | 0x0A | 0x0A |
| Byte | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| Contents | LEDCnt | Relay1T | Relay2T | BeepT | 0x00 | 0x00 | 0x00 |
| Default | 0x01 | 0x00 | 0x00 | 0x00 | 0x00 | 0x00 | 0x00 |
| Byte | 14 | 15 |  |  |  |  |  |
| Contents | 0x00 | 0x00 |  |  |  |  |  |
| Default | 0x00 | 0x00 |  |  |  |  |  |

BuzzOpenT: Buzzer on time, unit 10ms.

BuzzcloseT: Buzzer off time, unit 10ms.

BuzzCnt: Buzzer action count

BuzzVol: The volume of the buzzer. Value of 0 to 10, 0 to silent, 10 for the volume is the largest.

LEDEn: Action LED, The specified bit fix 1 can make the corresponding LED

Bit0: Channel 1 red.

Bit1: Channel 1 blue.

Bit2: Channel 2 red.

Bit3: Channel 2 blue.

LEDOpenT: LED on time, unit 100ms.

LEDcloseT: LED off time, unit 100ms.

LEDCnt: LED action count.

Relay1T: Relay 1 active time, unit 100ms.

Relay2T: Relay 2 active time, unit 100ms.

BeepT: onboard buzzer on time, unit 10ms.

- 6.2.11 CFG11: Backward Event

Define the action of the controller when person backward

| Byte | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contents | BuzzOpenT | BuzzcloseT | BuzzCnt | BuzzVol | LEDEn | LEDOpenT | LEDCloseT |
| Default | 0x00 | 0x00 | 0x00 | 0x00 | 0x0A | 0x0A | 0x0A |
| Byte | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| Contents | LEDCnt | Relay1T | Relay2T | BeepT | 0x00 | 0x00 | 0x00 |
| Default | 0x01 | 0x00 | 0x00 | 0x00 | 0x00 | 0x00 | 0x00 |
| Byte | 14 | 15 |  |  |  |  |  |
| Contents | 0x00 | 0x00 |  |  |  |  |  |
| Default | 0x00 | 0x00 |  |  |  |  |  |

BuzzOpenT: Buzzer on time, unit 10ms.

BuzzcloseT: Buzzer off time, unit 10ms.

BuzzCnt: Buzzer action count

BuzzVol: The volume of the buzzer. Value of 0 to 10, 0 to silent, 10 for the volume is the largest.

LEDEn: Action LED, The specified bit fix 1 can make the corresponding LED

Bit0: Channel 1 red.

Bit1: Channel 1 blue.

Bit2: Channel 2 red.

Bit3: Channel 2 blue.

LEDOpenT: LED on time, unit 100ms.

LEDcloseT: LED off time, unit 100ms.

LEDCnt: LED action count.

Relay1T: Relay 1 active time, unit 100ms.

Relay2T: Relay 2 active time, unit 100ms.

BeepT: onboard buzzer on time, unit 10ms.

- 6.2.12 CFG12: IR Trigger Enable

This configuration definition function of traps, controller only after trigger for infrared detection alarm or inventory tags

| Byte | 0 | 1 |
| --- | --- | --- |
| Contents | InfraredTrigEn | DelayTime |
| Default | 0x00 | 0x0A |

InfraredTrigEn: IR trigger enable flag,0-close; 1-open.

DelayTime: work time, unit 100ms.

- 6.2.13 CFG13: Tag filter Time

Under the inventory mode and emulated EAS alarm mode, read the same tag again in the filtering time do nothing

| Byte | 0 |
| --- | --- |
| Contents | FliterTime |
| Default | 0x01 |

FliterTime: filter time, unit 1S

- 6.2.14 CFG30: Inventory Type

This configuration is defined the read tag data types on inventory mode

| Byte | 0 |
| --- | --- |
| Contents | InventoryType |
| Default | 0x00 |

InventoryType:

0x00 –EPC;

0x01 –TID.

- 6.2.15 CFG31: Inventory Parameter

This configuration is defined the parameter on inventory mode

| Byte | 0 | 1 |
| --- | --- | --- |
| Contents | QValue | Session |
| Default | 0x04 | 0x00 |

QValue: Q Value, range is 0~15.

Session: Session Value

0x00: S0;

0x01: S1;

0x02: S2;

0x03: S3.

0xFF: Auto(only EPC).

- 6.2.16 CFG32: TID Parameter

This configuration is defined the TID parameter on inventory mode.

| Byte | 0 | 1 |
| --- | --- | --- |
| Contents | FirstAddr | Length |
| Default | 0x02 | 0x06 |

FirstAddr: Read TID first word,1Word=2Byte

Length: Read TID length, range is 1-15.

- 6.2.17 CFG33: Mask Set

This configuration is defined the Mask parameter on inventory mode.

| Byte | 0 | 1 | 2 | 3 | 4 | 5-36 |
| --- | --- | --- | --- | --- | --- | --- |
| Contents | MaskEn | MaskMem | MaskAddr |  | MaskLen | MaskData |
| Default | 0x00 | 0x01 | 0x00 | 0x00 | 0x00 | 0 |

MaskEn: Mask Enable bit. 0-disable;1-enable

MaskMem: mask memory,0x01-EPC;0x02-TID;0x03-User.

MaskAddr: Mask start address(unit bit),range is 0~16383.

MaskLen: Mask length(unit bit).

MaskData: Mask data.

- 6.2.18 CFG34: Work Region

This configuration is defined the RFID work region

| Byte | 0 | 1 | 2 |
| --- | --- | --- | --- |
| Contents | FreqBand | MaxFre | MinFre |
| Default | 0x02 | 0x31 | 0x00 |

FreqBand: RF band

0: reserve

1: Chinese band2 		Fs = 920.125 + N * 0.25 (MHz) N∈[0, 19].

2: US band				Fs = 902.75 + N * 0.5 (MHz) N∈[0,49].

3: Korean band			Fs = 917.1 + N * 0.2 (MHz) N∈[0, 31].

4: EU band				Fs = 865.1 + N*0.2(MHz) N∈[0, 14].

MaxFre: the Maximum frequency controller frequency points. The scope for the N

MinFre: the Minimum frequency controller frequency points. MinFre mast smaller than MaxFre.

- 6.2.19 CFG35: RF Power

This configuration is defined the power on antenna

| Byte | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contents | ANT1P | ANT2P | ANT3P | ANT4P | ANT5P | ANT6P | ANT7P | ANT8P |
| Default | 0x14 | 0x14 | 0x14 | 0x14 | 0x14 | 0x14 | 0x14 | 0x14 |

ANTnP: Antenna n Power output，range is 0-30, 30≈1W.

- 6.2.20 CFG36: Antenna

Used to define the effective antenna.

| Byte | 0 |
| --- | --- |
| Contents | ANT |
| Default | 0x0F |

ANT: effective antenna

Bit0:Channel 1 antenna 1, Bit1 Channel 1 antenna 2

Bit2:Channel 1 antenna 3, Bit3:Channel 1 antenna 4

Bit4:Channel 2 antenna 1, Bit5:Channel 2 antenna 2

Bit6:Channel 2 antenna 3, Bit7:Channel 2 antenna 4

- 6.2.21 CFG50: IP Address

This configuration is defined the network IP address

| Byte | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contents | IP-Address |  |  |  | SubnetMask |  |  |  |
| Default | 0xC0 | 0xA8 | 0x01 | 0xC0 | 0xFF | 0xFF | 0xFF | 0xFF |
| Byte | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| Contents | GW-Address |  |  |  | DHCP | 0x00 | 0x00 | 0x00 |
| Default | 0xC0 | 0xA8 | 0x01 | 0x01 | 0x00 | 0x00 | 0x00 | 0x00 |

IP-Address: Local IP address.

SubnetMask: Subnet mask address.

GW-Address:Gateway Address.

DHCP: DHCP Flag,0x00 – Disable;0x01 – Enable.

- 6.2.22 CFG51: Connection

This configuration is defined the network remote server IP and port.

| Byte | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contents | SERVERIP-Address |  |  |  | RemotePort |  | LocalPort |  | ConnectionMode |
| Default | 0xC0 | 0xA8 | 0x01 | 0xC0 | 0x2E | 0xE0 | 0x17 | 0x70 | 0x00 |

SERVERIP-Address: Remote Server IP Address when work as TCP Client mode. RemotePort: Remote Server IP Port when work as TCP Client mode.

LocalPort: Local Port when work as TCP Server mode.

ConnectionMode: TCP work mode, 0x00 – TCP Server; 0x01 –TCP Client .

- 6.2.23 CFG52: Repeat Connect

This configuration is defined the network repeat connect parameter

| Byte | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Contents | TCPTimeOut |  | TCPReconnectMode | 0x00 |
| Default | 0x02 | 0x58 | 0x00 | 0x00 |

TCPTimeOut: Controller as server, the continuous not to receive the client's data is still keep the connection time. High byte in the front. Unit: second. 0 means keep

TCPReconnectMode: The controller as the client, an active way of connecting to a remote server. 0 -connection immediately; 1 -have a data connection

- 6.2.24 CFG53: HeartBeat And Keepalive

This configuration is defined the network heartbeat and keep alive time

| Byte | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contents | HeartbeatEn | 0x00 | HeartbeatTime |  | keepaliveEn | 0x00 | keepaliveTime |  |
| Default | 0x00 | 0x00 | 0x00 | 0x3C | 0x01 | 0x00 | 0x00 | 0x0A |

HeartbeatEn: Heartbeat enable flag,0x00-disable, 0x01-enable.

HeartbeatTime: Heartbeat packet interval, the network free time is more than this time, will take the initiative to send a heartbeat packets. The heartbeat packet format defined in the table below.

keepaliveEn: Keep alive enable flag,0x00- disable, 0x01- enable.

keepaliveTime: Keepalive time interval, the network free time is more than this time, will take the initiative to send keepalive packet

Heartbeat packet upload format：

| Len | Addr | Cmd | Status | Data[] |  | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Time | PacketNo |  |  |
| 0x0F | 0xXX | 0xEF | 0x00 | 6bytes | 4bytes | LSB | MSB |

Time: Message produce time,Each byte followed by days, hours, minutes, seconds, milliseconds (high byte), ms (low byte). Days, hours, minutes and seconds the system 24 hours one day. Milliseconds for two bytes, the range of 0 ~ 999, the high byte in the former

PacketNo: Heartbeat package serial number. the high byte in the front. Controller sent up every heartbeat packets, a heartbeat packet number automatically add 1. If the heartbeat package serial number beyond 4 bytes, the serial number will back to 0 and start there

- 7．Network configuration

- 7.1 Network Parameter config

Controller supports two way of network parameters setting, for flexible use of customers.

1. The command config

Sent via RS232 or TCP interface, set the network parameters of mentioned above command, you can configure all network parameters of the controller.

2. Web Config

In the computer's web browser address bar enter the IP address of the device, can enter the configuration page of the controller.

- 7.2 Search Controller IP

Before the PC and controller in the network connection is established, must want to know the IP address of the controller, otherwise unable to establish a connection and setting up the network parameters. So we can provide a search within the current network controller IP address of the method.

Port 65535 is to read and write the specified UDP port radio, for writing device receives the data on this port if effective, will send the response data in the UDP broadcast way out. Because it is based on UDP broadcast communication, so packets can't across a router, can only communicate within the same physical link.

example: >>[255.255.255.255:65535] 58

<<[255.255.255.255:65535] C0 A8 01 02 00 7F 23 00 00 01

Send 255.255.255.255:65535 to data 0x58(Char‘X’), Get response data:C0 A8 01 02 00 7F 23 00 00 01, Means the IP 192.168.1.2, MAC:00-7F-23-00-00-01

Send data ’X’,Response:10byte.

IP(4byte)

MAC(6byte)

Send data’P’,Response: 25byte.

IP(4byte)

Subnet(4byte)

Gateway(4byte)

Server IP(4byte)

Server Port(2byte)

MAC(6byte)

Protocal(1byte)

When work as TCP Client, Server IP and Port is the remote.

When work as TCP Server, Server IP and Port is the local.

Protocal : 0=TCP server, 1= TCP client, 2= UDP.