UHFGate.dll manual V1.0

1.Operation System Requirement	2

2.Function List	2

3.Function Explanation	5

3.1) AutoOpenComPort	5

3.2) OpenComPort	5

3.3) CloseComPort	6

3.4) CloseSpecComPort	6

3.5) OpenNetPort	7

3.6) CloseNetPort	7

3.7) GetControllerInfo	7

3.8) ConfigureController	8

3.9) GetControllerConfig	8

3.10) GetChannelMessage	8

3.11) Acknowledge	10

3.12) SetRelay	10

3.13) SetBuzzer	10

3.14) SetLED	11

3.15) GetClock	11

3.16) SetClock	11

3.17) ClearBuffer	12

3.18) GetPersionAndAlarmInfo	12

3.19) SetPersionAndAlarmInfo	12

3.20) SetGPIO	13

3.21) GetGPIO	13

3.22) GetControllerReaderConnectionStatus	13

3.23) ControllerRestart	14

3.24) LoadDefault	14

3.25) GetDeviceMAC	14

3.26) GetRerurnLoss	15

4.Parameter config	15

4.1 Controller configuration parameters table	15

4.2 Controller configuration parameters function	17

4.2.1 CFG1：Address	17

4.2.2 CFG2：Work Mode	17

4.2.3 CFG3：Alarm mode	17

4.2.4 CFG4：Event Enable	18

4.2.5 CFG5：Custom alarm rule	18

4.2.6 CFG6：Alarm Event action	19

4.2.7 CFG7：Message	19

4.2.8 CFG8：Message Transmission	20

4.2.9 CFG9：Person direction	22

4.2.10 CFG10：Person forward enent	22

4.2.11 CFG11：Person backward event	23

4.2.12 CFG12：IR trigger enable	24

4.2.13 CFG13：Tag filter Time	24

4.2.14 CFG30：Inventory Type	24

4.2.15 CFG31：Inventory Param	25

4.2.16 CFG32：TID Param	25

4.2.17 CFG33：Mask Set	25

4.2.18 CFG34：Region	26

4.2.19 CFG35：Power	26

4.2.20 CFG36：Antenna	26

4.2.21 CFG50：IP Address	27

4.2.22 CFG51：Connection	27

4.2.23 CFG52：Repeat Connect	27

4.2.24 CFG53：HeartBeat and keepalive	28

4.2.25 CFG14：Cascade door set	28

5.Append	29

# 1.Operation System Requirement

WINDOWS 2000/XP/7/8/10

# 2.Function List

UHFGATE.dll includes the following functions:

1) int OpenNetPort(int Port, char* IPaddr, unsigned char*ComAdr, int *FrmHandle);

2) int CloseNetPort(int FrmHandle);

3) int OpenComPort(int port,unsigned char *address,int* FrmHandle);

4) int AutoOpenComPort(int *port,unsigned char *address,int *FrmHandle);

5) int CloseComPort();

6) int CloseSpecComPort(int FrmHandle);

7) int GetControllerInfo(unsigned char* ComAdr,unsigned char* ProductCode,unsigned char* Version,unsigned char *DevSN,int FrmHandle);

8) int ConfigureController(unsigned char ComAdr,unsigned char CFGNo,unsigned char CFGLen,unsigned char* CFGData,int FrmHandle);

9) int GetControllerConfig(unsigned char ComAdr,unsigned char CFGNo,unsigned char *CFGLen,unsigned char* CFGData,int FrmHandle);

10) int GetChannelMessage(unsigned char ComAdr,unsigned char* PacketNo, unsigned char* Msg, unsigned char* MsgLength, unsigned char* MsgType,int FrmHandle);

11) int Acknowledge (unsigned char ComAdr,int FrmHandle);

12) int SetRelay(unsigned char ComAdr,unsigned char num,unsigned char status,unsigned char time,int FrmHandle);

13) int SetBuzzer(unsigned char ComAdr,unsigned char onTime,unsigned char offTime,unsigned char Count, unsigned char RFU,int FrmHandle);

14) int SetLED(unsigned char ComAdr,unsigned char onTime,unsigned char offTime,unsigned char Count, unsigned char Color,int FrmHandle);

15) int GetClock(unsigned char ComAdr,unsigned char* CurTime,int FrmHandle);

16) int SetClock(unsigned char ComAdr,unsigned char* SetTime,int FrmHandle);

17) int ClearBuffer(unsigned char ComAdr,int FrmHandle);

18) int GetPersionAndAlarmInfo(unsigned char ComAdr,unsigned char* positive,unsigned char* reverse,unsigned char* Online,unsigned char* Alarm,int FrmHandle);

19) int SetPersionAndAlarmInfo(unsigned char ComAdr,unsigned char* positive,unsigned char* reverse,unsigned char* Online,unsigned char* Alarm,int FrmHandle);

20) int SetGPIO(unsigned char ComAdr,unsigned char GPIO_OUT,int FrmHandle);

21) int GetGPIO(unsigned char ComAdr,unsigned char* GPI,int FrmHandle);

22)int GetControllerReaderConnectionStatus(unsigned char ComAdr,unsigned char* ConnectStatus,int FrmHandle);

23) int ControllerRestart(unsigned char ComAdr,int FrmHandle);

24) int LoadDefault(unsigned char ComAdr,int FrmHandle);

25) int ReadActiveModeData(unsigned char ComAdr,unsigned char*Data,int *Len,int FrmHandle);

26) int GetRerurnLoss(unsigned char ComAdr,unsigned char antenna,unsigned char*ReturnLoss,int FrmHandle);

# 3.Function Explanation

## 3.1) AutoOpenComPort

int AutoOpenComPort(int* Port, unsigned char *ConAddr, int* FrmHandle);

Function description：This function is used to automatically detect the communication port unoccupied by other application and attached with a reader.。

Parameter：

Port：Pointed to the communication port number(COM1~COM12) that the reader is detected and connected.

Constants COM1~COM12 are defined as follows:

#define COM1  1

#define COM2  2

#define COM3  3

#define COM4  4

#define COM5  5

#define COM6  6

#define COM7  7

#define COM8  8

#define COM9  9

#define COM10  10

#define COM11  11

#define COM12  12

ConAddr：Pointed to the address of the controller.

FrmHandle：Pointed to the communication handle which is binding with the communication port opened successfully. The application software should use this handle to manipulate the reader connected to the port

Returns:

Zero value when successful, non-zero value(ErrorCode) when error occurred.。

## 3.2) OpenComPort

int OpenComPort(int Port,  unsigned char *ConAddr, int *FrmHandle);

Function description：This function is used to establish the connection between the reader and a specified communication port.

Parameter：

Port：Communication port number which is a constant from COM1 to COM12 defined as following:

#define COM1  1

#define COM2  2

#define COM3  3

#define COM4  4

#define COM5  5

#define COM6  6

#define COM7  7

#define COM8  8

#define COM9  9

#define COM10  10

#define COM11  11

#define COM12  12

ConAddr：Pointed to the address of the controller.

FrmHandle：Pointed to the communication handle which is binding with the communication port opened successfully. The application software should use this handle to manipulate the reader connected to the port

Returns:

Zero value when successful, non-zero value(ErrorCode) when error occurred.。

## 3.3) CloseComPort

int CloseComPort( void);

Function description:

This function is used to disconnect the reader and release the corresponding communication port resources. In some development environment, the communication port resources must be released before exiting. Otherwise the operation system will become unstable.

Parameter: None.

Returns:

Zero value when successful, non-zero value(ErrorCode) when error occurred.

## 3.4) CloseSpecComPort

int CloseSpecComPort(int FrmHamdle);

Function description:

This function is used to disconnect the reader with the designated communication port and release the corresponding resources.参数：

Parameter:

FrmHandle: Handle of the corresponding communication.

Returns:

Zero value when successful, non-zero value(ErrorCode) when error occurred.

## 3.5) OpenNetPort

int OpenNetPort(int Port,  char* IPaddr,unsigned char *ConAddr, int *FrmHandle);

Function description:

This function is used to used to open net port.

Parameter:

Port: Input. Pointed to the net port of the reader.

IPaddr: Input: Pointed to string of reader IP.

ConAddr: Input/Output. Pointed to the address of the reader.

When using broadcasting address 0xFF as ComAddr to call the function, the address of the reader will be written back to parameter ComAddr;

When using a designated address 0x00~0xFE as ComAddr to call the function, the function will detect whether a specified address reader is connected to the designated communication port.

FrmHandle: Output. Pointed to the handle which is binding with the communication port opened successfully. The application software should use this handle to talk with the reader.

Returns:

Zero value when successful, non-zero value when error occurred

## 3.6) CloseNetPort

int CloseNetPort(int FrmHandle);

Function description:

This function is used to disconnect net port

Parameter：

FrmHandle：Input. Handle of the communication.

Returns:

Zero value when successful, non-zero value when error occurred

## 3.7) GetControllerInfo

int GetControllerInfo(unsigned char* ComAdr,unsigned char* ProductCode,unsigned char* Version,unsigned char *DevSN,int FrmHandle);

Function description:

Use this command to get controller system version information.

Parameter：

ComAddr：input and output，1 byte，Controller address.

ProductCode：output，1byte，controller code

Version：output,2 bytes，Controller version

DevSN：output，4 bytes, Controller device id.

FrmHandle：Input. Handle of the communication.

Returns:

Zero value when successful, non-zero value when error occurred

## 3.8) ConfigureController

ConfigureController(unsigned char ComAdr,unsigned char CFGNo,unsigned char CFGLen,unsigned char* CFGData,int FrmHandle);

Function description:

Used to configure the parameters of the controller

Parameter：

ComAddr：input，1 byte，Controller address.

CFGNo：input，Used to specify to configure ID.

CFGLen：input, the length of CFGData.

CFGData：input，the data of configure，see Append 1。

FrmHandle：Input. Handle of the communication.

Returns:

Zero value when successful, non-zero value when error occurred

## 3.9) GetControllerConfig

GetControllerConfig (unsigned char ComAdr,unsigned char CFGNo,unsigned char *CFGLen,unsigned char* CFGData,int FrmHandle);

Function description:

Used to get the parameters of the controller

Parameter：

ComAddr：input，1 byte，Controller address.

CFGNo：input，Used to specify to configure ID.

CFGLen：output, the length of CFGData.

CFGData：output，the data of configure，see Append 1

FrmHandle：Input. Handle of the communication.

Returns:

Zero value when successful, non-zero value when error occurred

## 3.10) GetChannelMessage

int GetChannelMessage(unsigned char ComAdr,unsigned char* PacketNo, unsigned char* Msg, unsigned char* MsgLength, unsigned char* MsgType,int FrmHandle);

Function description:

get message from controller about UID and relevant information。

Parameter：

ComAddr：input，1 byte，Controller address.

PacketNo：output，1 byte，Message number. Each send a message, the message sequence number automatic one, beyond the scope of a byte will back to 0 to accumulate. Used to distinguish between new messages and repeat。

Msg：output，Message data

MsgLength：output，1 byte，the length of Msg.

MsgType：output，1byte，Message Type。

(1)  MsgType = 0，null message, MsgLength=0。

(2)  MsgType = 1，Inventory message：Msg 1-6 bytes, Each byte followed by days, hours, minutes, seconds, milliseconds (high byte), ms (low byte). Days, hours, minutes and seconds the system 24 hours a day. Milliseconds for two bytes, the range of 0 ~ 999, the high byte in the former,7 byte is tag number and other bytes are EPC/TID (length and EPC/TID)

(3)  MsgType = 2，EAS Alarm Message：Msg 1-6 byte, Each byte followed by days, hours, minutes, seconds, milliseconds (high byte), ms (low byte). Days, hours, minutes and seconds the system 24 hours a day. Milliseconds for two bytes, the range of 0 ~ 999, the high byte in the former,7 byte is alarm type ,0-no alarm；1-EAS Alarm(NXP)；2-Emulate EAS alarm，other bytes are EPC/TID (length and EPC/TID).

(4)  MsgType = 3，Person Message：Msg 1-6 bytes, Each byte followed by days, hours, minutes, seconds, milliseconds (high byte), ms (low byte). Days, hours, minutes and seconds the system 24 hours a day. Milliseconds for two bytes, the range of 0 ~ 999, the high byte in the former,7 byte is direction。

0：forward

1，backward

2，indefinite.

8~11 bytes is forward number，12~15 bytes is backward number，16~19 bytes is online number.

(5)  MsgType = 4，IR Message：Msg 1-6 bytes, Each byte followed by days, hours, minutes, seconds, milliseconds (high byte), ms (low byte). Days, hours, minutes and seconds the system 24 hours a day. Milliseconds for two bytes, the range of 0 ~ 999, the high byte in the former,7 byte is IR line information 。

0：line 1 first IR；

1：line 1 second IR；

2：line 2 first IR；

3：line 2 second IR；

8 byts is IR status，0-block；1-sync

FrmHandle：Input. Handle of the communication.

Returns:

Zero value when successful, non-zero value when error occurred

## 3.11) Acknowledge

int Acknowledge (unsigned char ComAdr,int FrmHandle);

Function description:

Confirm the PC correct accept information sent to the controller.

Parameter：

ComAddr：input，1 byte，Controller address.

FrmHandle：Input. Handle of the communication.

Returns:

null

## 3.12) SetRelay

int SetRelay(unsigned char ComAdr,unsigned char num,unsigned char status,unsigned char time,int FrmHandle);

Function description:

Use this command to control relay

Parameter：

ComAddr：input，1 byte，Controller address.

num：input ,control the relay number, 0 - relay 1; 1 - relay 2

status：input ,0x00 -release，0x01-active

time：input ,Relay Time (Time * 100 ms), absorption when the Status of 0 x01 this parameter. And time to automatically after release. If this parameter is 0, says does not automatically release, has been attracting.

FrmHandle：Input. Handle of the communication.

Returns:

Zero value when successful, non-zero value when error occurred

## 3.13) SetBuzzer

int SetBuzzer(unsigned char ComAdr,unsigned char onTime,unsigned char offTime,unsigned char Count, unsigned char Vol,int FrmHandle);

Function description:

Use this command to control buzzer.

Parameter：

ComAddr：input，1 byte，Controller address.

onTime：input，beep on time，unit 10ms。

offTime：input，beep off time, unit 10ms。

Count：input，active times.

Vol：input，The volume and value of 0 to 10. 0 to mute, 10 volume is the largest.

FrmHandle：Input. Handle of the communication.

Returns:

Zero value when successful, non-zero value when error occurred

## 3.14) SetLED

int SetLED(unsigned char ComAdr,unsigned char onTime,unsigned char offTime,unsigned char Count, unsigned char Color,int FrmHandle);

Function description:

Use this command to control the access door at the top of the LED lights

Parameter：

ComAddr：input，1 byte，Controller address.

onTime：input，LED on time，unit is 100ms。

offTime：input，LED off time, unit is 100ms。

Count：input，LED flash times.

Color：input，LED on color.

Bit0 channel 1 red

Bit1 channel 1 blue

Bit2 channel 2 red

Bit3 channel 2 blue

FrmHandle：Input. Handle of the communication.

Returns:

Zero value when successful, non-zero value when error occurred

## 3.15) GetClock

int GetClock(unsigned char ComAdr,unsigned char* CurTime,int FrmHandle);

Function description:

Used to read the current time of the controller internal real time clock.

Parameter：

ComAddr：input，1 byte，Controller address.

CurTime：output，6 bytes，From left to right, each byte order for the year, month, day, hours, minutes and seconds. System time format for 24 hours

FrmHandle：Input. Handle of the communication.

Returns:

Zero value when successful, non-zero value when error occurred

## 3.16) SetClock

int SetClock(unsigned char ComAdr,unsigned char* CurTime,int FrmHandle);

Function description:

Used to read the current time of the controller internal real time clock.

Parameter：

ComAddr：input，1 byte，Controller address.

CurTime：input，6 bytes，From left to right, each byte order for the year, month, day, hours, minutes and seconds. System time format for 24 hours

FrmHandle：Input. Handle of the communication.

Returns:

Zero value when successful, non-zero value when error occurred

## 3.17) ClearBuffer

int ClearBuffer(unsigned char ComAdr,int FrmHandle);

Function description:

Send the command to clear the controller message

Parameter：

ComAddr：input，1 byte，Controller address.

FrmHandle：Input. Handle of the communication.

Returns:

Zero value when successful, non-zero value when error occurred

## 3.18) GetPersionAndAlarmInfo

int GetPersionAndAlarmInfo(unsigned char ComAdr,unsigned char* positive,unsigned char* reverse,unsigned char* Online,unsigned char* Alarm,int FrmHandle);

Function description:

Used to read the controller in and out of the number of statistical data and alarm system。

Parameter：

ComAddr：input，1 byte，Controller address.

positive：output，4 bytes，forward number，High byte in the front.

reverse：output，4 bytes，backward number，High byte in the front.

Online：output，4 bytes，online number，High byte in the front.

Alarm：output，4 bytes，Alarm number，High byte in the front.

FrmHandle：Input. Handle of the communication.

Returns:

Zero value when successful, non-zero value when error occurred

## 3.19) SetPersionAndAlarmInfo

int GetPersionAndAlarmInfo(unsigned char ComAdr,unsigned char* positive,unsigned char* reverse,unsigned char* Online,unsigned char* Alarm,int FrmHandle);

Function description:

Used to read the controller in and out of the number of statistical data and alarm system。

Parameter：

ComAddr：input，1 byte，Controller address.

positive：output，4 bytes，forward number，High byte in the front.

reverse：output，4 bytes，backward number，High byte in the front.

Online：output，4 bytes，online number，High byte in the front.

Alarm：output，4 bytes，Alarm number，High byte in the front.

FrmHandle：Input. Handle of the communication.

Returns:

Zero value when successful, non-zero value when error occurred

## 3.20) SetGPIO

int SetGPIO(unsigned char ComAdr,unsigned char GPIO_OUT,int FrmHandle);

Function description:

Use this command to control the state of the GPO output。

Parameter：

ComAddr：input，1 byte，Controller address.

GPIO_OUT：input，GPIO status，bit3-bit0 means Out4~Out1,0-null，1-high.

FrmHandle：Input. Handle of the communication.

Returns:

Zero value when successful, non-zero value when error occurred

## 3.21) GetGPIO

int GetGPIO(unsigned char ComAdr,unsigned char* GPIO_OUT,int FrmHandle);

Function description:

Use this command to read the state of the GPI input。

Parameter：

ComAddr：input，1 byte，Controller address.

GPIO_OUT：input，GPIO status，bit3-bit0 means input4~ input1,0-null，1-high.

FrmHandle：Input. Handle of the communication.

Returns:

Zero value when successful, non-zero value when error occurred

## 3.22) GetControllerReaderConnectionStatus

int GetControllerReaderConnectionStatus(unsigned char ComAdr,unsigned char* ConnectStatus,int FrmHandle);

Function description:

Used to check the communication between controller and reader.

Parameter：

ComAddr：input，1 byte，Controller address.

ConnectStatus：output，1 byte，connect status，bit0-reader1，bit4-reader2,0-disconnect，1-connect.

FrmHandle：Input. Handle of the communication.

Returns:

Zero value when successful, non-zero value when error occurred

## 3.23) ControllerRestart

int ControllerRestart(unsigned char ComAdr,int FrmHandle);;

Function description：

Use this command to restart controller.

Parameter：

ComAddr：input，1 byte，Controller address.

FrmHandle：Input. Handle of the communication.

Returns:

Zero value when successful, non-zero value when error occurred

## 3.24) LoadDefault

int LoadDefault(unsigned char ComAdr,int FrmHandle);

Function description：

Used the command to load the default parameters.

Parameter：

ComAddr：input，1 byte，Controller address.

FrmHandle：Input. Handle of the communication.

Returns:

Zero value when successful, non-zero value when error occurred

## 3.25) GetDeviceMAC

int GetDeviceMAC(unsigned char ComAdr,unsigned char*MacData,int FrmHandle);

Function description：

Used this command to get the device network MAC address

Parameter：

ComAddr：input，1 byte，Controller address.

MacData：output，6 bytes，mac address

FrmHandle：Input. Handle of the communication.

Returns:

Zero value when successful, non-zero value when error occurred

## 3.26) GetRerurnLoss

int GetRerurnLoss(unsigned char ComAdr,unsigned char antenna,unsigned char*ReturnLoss,int FrmHandle);;

Function description：

Used to get the antenna return loss measurement。

Parameter：

ComAddr：input，1 byte，Controller address.

antenna：input，0-7 means antenna1~antenna8.

ReturnLoss：output，1 byte, return loss

FrmHandle：Input. Handle of the communication.

Returns:

Zero value when successful, non-zero value when error occurred

# 4.Parameter config

All the control parameters of the controller are listed in this section, the convenient user unified set and read.

Parameter read using command reads the controller parameters, the parameter Settings using the controller configuration commands. Set up after the command, the configuration will not take effect immediately, you need to use the save application configuration commands make command to save and take effect. If the parameters are stored before controller was shut down or restart, has set the parameters will be lost, and use the last right to save the parameters of the work.

## 4.1 Controller configuration parameters table

configuration table：

| No. | Name | Length(byte) | describe |
| --- | --- | --- | --- |
| 1 | Address | 1 | Controller Address |
| 2 | Work Mode | 1 | Work mode: Inventory mode ,EAS Mode |
| 3 | Alarm Mode | 1 | Alarm mode: EAS、emulate EAS |
| 4 | Event enable | 4 | Various events enabled, such as open alarm events。 |
| 5 | Custom alarm rule | 128 | According to the Bit mask way custom alarm rules |
| 6 | Alarm event action | 16 | controller action When alarming, such as action buzzer, LED lights |
| 7 | Message | 4 | The message type, such as messages, traps of personnel entering and leaving |
| 8 | Message transmission | 2 | Message transmission and validation mechanism |
| 9 | Person direction | 1 | Define into and out of the direction |
| 10 | Person into event | 16 | the action of the controller when person forward |
| 11 | Person out even | 16 | the action of the controller when person backward |
| 12 | IR trigger enable | 2 | IR trigger function |
| 13 | tag filter time | 1 | A single tag filtering time |
| 14 | Cascade door set | 4 | Unicom gate level associated Settings |
| … | … |  |  |
| 30 | Inventory Type | 1 | Inventory EPC or TID |
| 31 | Inventory Param | 2 | Q and Session |
| 32 | TID Param | 2 | Set the start address and length |
| 33 | Mask set | 37 | Inventory mask set |
| 34 | region | 3 | Set rfid work region |
| 35 | Power | 8 | Set RFID work power |
| 36 | Antenna | 1 | Set effective antenna |
| … | … |  |  |
| 50 | IP address | 13 | Set the controller ip information |
| 51 | Connection | 8 | Set the TCP mode, remote IP and port information |
| 52 | Repeat connect | 4 | Set up the network timeout disconnect and bolt connection |
| 53 | Heartbeat and keepalive | 4 | set heartbeat and keepalive parameter |

## 4.2 Controller configuration parameters function

### 4.2.1 CFG1：Address

This configuration is defined the correspondence address of the controller.

For the controller is not native or address 0 xFF command, the controller does not respond.

| Byte | 0 |
| --- | --- |
| Contents | Addr |
| Default | 0x00 |

Addr: address, range is 0x00-0xFE。

### 4.2.2 CFG2：Work Mode

Controller supports two work mode, inventory mode and EAS mode.

Inventory mode used to query multiple tag EPC/TID. Will not trigger the alarm.

EAS mode used to detect alarm, get in line with the alarm tag will send alarm signal

| Byte | 0 |
| --- | --- |
| Contents | WorkMode |
| Default | 0x00 |

WorkMode：0x00 Inventory mode.

0x01 EAS mode

### 4.2.3 CFG3：Alarm mode

In EAS mode, you can set different alarm mode, meet different application scenarios。

| Byte | 0 | 1 |
| --- | --- | --- |
| Contents | AlarmMode | AlarmType |
| Default | 0x00 | 0x00 |

AlarmMode: 0x00 EAS Alarm(NXP)

0x01 Emulated EAS Alarm

AlarmType： when set Emulated EAS Alarm，this parameter define alarm type\.

0x00: Mode A, EPC 92、93bit value is 01alarm.

0x01: Mode B, EPC first bit is 0 alarm.

0x02: Mode C, alarm when detect one tag.

0x03:Mode D, Custom alarm rules. See 4.2.5

### 4.2.4 CFG4：Event Enable

Enable event, when enabled，actions as follows:

| Byte | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Contents | EventEn | 0x00 | 0x00 | 0x00 |
| Default | 0x01 | 0x00 | 0x00 | 0x00 |

EventEn:1 byte。

Bit0:Alarm event

Bit1:Forward event

Bit2:Backward event

### 4.2.5 CFG5：Custom alarm rule

Custom rules support Bit mask set alarm rules.

Controller working in emulated EAS alarm mode, and select the custom EAS alarm, after the success for tag number of EPC, preferred to judge whether the length of the EPC between MinLen and MaxLen, does not conform to the discarded directly, to conform to the EPC, it Bit by Bit from the top byte EPC to mask the comparison, the check corresponding MaskEn buy 1 Bit in the EPC number with corresponding Bit MaskData values are the same, different is discarded the EPC number, all the same just call the police.

| Byte | 0-61 | 62-123 | 124 | 125 | 126 | 127 |
| --- | --- | --- | --- | --- | --- | --- |
| Contents | MaskEn | MaskData | MinLen | MaxLen | 0x00 | 0x00 |
| Default | 0x00 | 0x00 | 0x00 | 0x | 0x00 | 0x00 |

MaskEn: Mask Enable bit.

MaskData：Mask data.

MinLen：Minimum EPC number length.

MaxLen：Maximum EPC number length.

### 4.2.6 CFG6：Alarm Event action

controller action When alarming, such as action buzzer, LED lights

| Byte | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contents | BuzzOpenT | BuzzcloseT | BuzzCnt | BuzzVol | LEDEn | LEDOpenT | LEDCloseT |
| Default | 0x00 | 0x00 | 0x00 | 0x00 | 0x00 | 0x00 | 0x00 |
| Byte | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| Contents | LEDCnt | Relay1T | Relay2T | BeepT | 0x00 | 0x00 | 0x00 |
| Default | 0x00 | 0x00 | 0x00 | 0x00 | 0x00 | 0x00 | 0x00 |
| Byte | 14 | 15 |  |  |  |  |  |
| Contents | 0x00 | 0x00 |  |  |  |  |  |
| Default | 0x00 | 0x00 |  |  |  |  |  |

BuzzOpenT：Buzzer on time, unit 10ms.

BuzzcloseT：Buzzer off time, unit 10ms.

BuzzCnt：Buzzer action count.

BuzzVol：The volume of the buzzer. Value of 0 to 10, 0 to silent, 10 for the volume is the largest.

LEDEn：Action LED, The specified bit fix 1 can make the corresponding LED

Bit0: Channel 1 red.

Bit1: Channel 1 blue.

Bit2: Channel 2 red.

Bit3: Channel 2 blue.

LEDOpenT：LED on time, unit 100ms.

LEDcloseT：LED off time, unit 100ms.

LEDCnt：LED action count.

Relay1T：Relay 1 active time, unit 100ms.

Relay2T：Relay 2 active time, unit 100ms.

BeepT：onboard buzzer on time, unit 10ms。

### 4.2.7 CFG7：Message

The generation of this configuration controller messages defined.

| Byte | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Contents | MessageEn | 0x00 | 0x00 | 0x00 |
| Default | 0x01 | 0x00 | 0x00 | 0x00 |

MessageEn: According to the bit define which information allows to upload.

Bit0:Person message

Bit1:IR Message

Bit2:No Alarm tag Message

### 4.2.8 CFG8：Message Transmission

This configuration is defined controller message transmission and confirm the mechanism

| Byte | 0 | 1 |
| --- | --- | --- |
| Contents | MsgAutoUpload | MsgAck |
| Default | 0x00 | 0x01 |

MsgAutoUpload：News initiative to upload. 1- auto upload, 0-no active upload need to send the GetMessage command to get the message.

MsgAck：Need ACK flag, 0- no ACK，1-need ACK。

Auto upload data format：

| Len | Addr | Cmd | Status | Data[] |  |  | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xXX | 0xXX | 0xEE | 0x00 | DevSN | MsgType | Data | LSB | MSB |
|  |  |  |  | 4 Byte | 0xXX | n Byte |  |  |

DevSN: 4 bytes，device ID,

MsgType: Message Type：

0x00：null, Data[] is null。

0x01: Inventory Message

0x02: EAS Alarm Message

0x03: Person Message

0x04: IR Message

Data：n Byte

(1) Inventory Message

| Data[] |  |  |
| --- | --- | --- |
| Time | Num | EPC-1, |
| 6bytes | 1byte | N bytes |

Time：The message of time. From left to right, each byte followed by days, hours, minutes, seconds, milliseconds (high byte), ms (low byte). USES the system 24 hours a day. Milliseconds for two bytes, the range of 0 ~ 999, the high byte in the front.

Num: tag number.

EPC ID: The inquired tag data, the data block follows the format stated below:

EPC/TID length + EPC/TID No.

(2) EAS Alarm Message

| Data[] |  |  |
| --- | --- | --- |
| Time | EASType | EPC ID |
| 6bytes | 1byte | n bytes |

Time：The message of time. From left to right, each byte followed by days, hours, minutes, seconds, milliseconds (high byte), ms (low byte). USES the system 24 hours a day. Milliseconds for two bytes, the range of 0 ~ 999, the high byte in the front.

EASType：EAS Alarm Type.0-no EAS alarm；1-EAS alarm(NXP)；2-Emulated EAS alarm.

EPC ID：Alarm tag EPC。

Note: the data of EPCID only when Emulated EAS alarm set to have a return. Standard EAS alarm no this field。

(3) Person Message

| Data[] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Time | In/Out | positive | reverse | Online |
| 6bytes | 0xXX | 4bytes | 4bytes | 4bytes |

Time：The message of time. From left to right, each byte followed by days, hours, minutes, seconds, milliseconds (high byte), ms (low byte). USES the system 24 hours a day. Milliseconds for two bytes, the range of 0 ~ 999, the high byte in the front.

In/Out：Current direction.

0：Forward

1，Backward

2，indefinite.

Positive：Forward number

Reverse：Backward number

Online：onLine number

(4)IR Message

| Data[] |  |  |
| --- | --- | --- |
| Time | Channel | Active |
| 6bytes | 1byte | 1byte |

Time：The message of time. From left to right, each byte followed by days, hours, minutes, seconds, milliseconds (high byte), ms (low byte). USES the system 24 hours a day. Milliseconds for two bytes, the range of 0 ~ 999, the high byte in the front.

Channel：the IR line

0：the line 1 of Channel 1

1：the line 2 of Channel 1

2：the line 1 of Channel 2

3：the line 2 of Channel 2

Active: IR status，0-block；1-Sync

### 4.2.9 CFG9：Person direction

Define into and out of the direction

| Byte | 0 |
| --- | --- |
| Contents | Direction |
| Default | 0x00 |

Direction： 0x00-forward，0x01-backward

### 4.2.10 CFG10：Person forward enent

the action of the controller when person forward

| Byte | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contents | BuzzOpenT | BuzzcloseT | BuzzCnt | BuzzVol | LEDEn | LEDOpenT | LEDCloseT |
| Default | 0x00 | 0x00 | 0x00 | 0x00 | 0x00 | 0x00 | 0x00 |
| Byte | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| Contents | LEDCnt | Relay1T | Relay2T | BeepT | 0x00 | 0x00 | 0x00 |
| Default | 0x00 | 0x00 | 0x00 | 0x00 | 0x00 | 0x00 | 0x00 |
| Byte | 14 | 15 |  |  |  |  |  |
| Contents | 0x00 | 0x00 |  |  |  |  |  |
| Default | 0x00 | 0x00 |  |  |  |  |  |

BuzzOpenT：Buzzer on time, unit 10ms。

BuzzcloseT：Buzzer off time, unit 10ms。

BuzzCnt：Buzzer action count

BuzzVol：The volume of the buzzer. Value of 0 to 10, 0 to silent, 10 for the volume is the largest.

LEDEn：Action LED, The specified bit fix 1 can make the corresponding LED

Bit0: Channel 1 red.

Bit1: Channel 1 blue.

Bit2: Channel 2 red.

Bit3: Channel 2 blue.

LEDOpenT：LED on time, unit 100ms.

LEDcloseT：LED off time, unit 100ms.

LEDCnt：LED action count.

Relay1T：Relay 1 active time, unit 100ms.

Relay2T：Relay 2 active time, unit 100ms.

BeepT：onboard buzzer on time, unit 10ms。

### 4.2.11 CFG11：Person backward event

the action of the controller when person backward

| Byte | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contents | BuzzOpenT | BuzzcloseT | BuzzCnt | BuzzVol | LEDEn | LEDOpenT | LEDCloseT |
| Default | 0x00 | 0x00 | 0x00 | 0x00 | 0x00 | 0x00 | 0x00 |
| Byte | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| Contents | LEDCnt | Relay1T | Relay2T | BeepT | 0x00 | 0x00 | 0x00 |
| Default | 0x00 | 0x00 | 0x00 | 0x00 | 0x00 | 0x00 | 0x00 |
| Byte | 14 | 15 |  |  |  |  |  |
| Contents | 0x00 | 0x00 |  |  |  |  |  |
| Default | 0x00 | 0x00 |  |  |  |  |  |

BuzzOpenT：Buzzer on time, unit 10ms。

BuzzcloseT：Buzzer off time, unit 10ms。

BuzzCnt：Buzzer action count

BuzzVol：The volume of the buzzer. Value of 0 to 10, 0 to silent, 10 for the volume is the largest.

LEDEn：Action LED, The specified bit fix 1 can make the corresponding LED

Bit0: Channel 1 red.

Bit1: Channel 1 blue.

Bit2: Channel 2 red.

Bit3: Channel 2 blue.

LEDOpenT：LED on time, unit 10ms.

LEDcloseT：LED off time, unit 10ms.

LEDCnt：LED action count.

Relay1T：Relay 1 active time, unit 100ms.

Relay2T：Relay 2 active time, unit 100ms.

BeepT：onboard buzzer on time, unit 10ms。

### 4.2.12 CFG12：IR trigger enable

This configuration definition function of traps, controller only after trigger for infrared detection alarm or inventory tags

| Byte | 0 | 1 |
| --- | --- | --- |
| Contents | InfraredTrigEn | DelayTime |
| Default | 0x00 | 0x0A |

InfraredTrigEn：IR trigger enable flag,0-close，1-open

DelayTime：work time, unit 100ms。

### 4.2.13 CFG13：Tag filter Time

Under the inventory mode and emulated EAS alarm mode, read the same tag again in the filtering time do nothing

| Byte | 0 |
| --- | --- |
| Contents | FliterTime |
| Default | 0x01 |

FliterTime：filter time, unit 1S

### 4.2.14 CFG30：Inventory Type

This configuration is defined the read tag data types on inventory mode

| Byte | 0 |
| --- | --- |
| Contents | InventoryType |
| Default | 0x00 |

InventoryType：0x00-EPC，0x01-TID。

### 4.2.15 CFG31：Inventory Param

This configuration is defined the parameter on inventory mode

| Byte | 0 | 1 |
| --- | --- | --- |
| Contents | QValue | Session |
| Default | 0x04 | 0x00 |

QValue：Q Value, range is 0~15.

Session：Session Value

0x00：S0；

0x01：S1；

0x02：S2；

0x03：S3。

0xff：Auto（only EPC）.

### 4.2.16 CFG32：TID Param

This configuration is defined the TID parameter on inventory mode

| Byte | 0 | 1 |
| --- | --- | --- |
| Contents | FirstAddr | Length |
| Default | 0x00 | 0x06 |

FirstAddr：Read TID first word,1Word=2Byte。

Length：Read TID length, range is 1-15。

### 4.2.17 CFG33：Mask Set

This configuration is defined the Mask parameter on inventory mode

| Byte | 0 | 1 | 2 | 3 | 4 | 5-36 |
| --- | --- | --- | --- | --- | --- | --- |
| Contents | MaskEn | MaskMem | MaskAddr |  | MaskLen | MaskData |
| Default | 0x00 | 0x01 | 0x00 | 0x00 | 0x00 | 0 |

MaskEn: Mask Enable bit. 0-disable;1-enable

MaskMem：mask memory,0x01-EPC;0x02-TID;0x03-User.

MaskAddr：Mask start address(unit bit),range is 0~16383.

MaskLen：Mask length(unit bit).

MaskData：Mask data.

### 4.2.18 CFG34：Region

This configuration is defined the RFID work region

| Byte | 0 | 1 | 2 |
| --- | --- | --- | --- |
| Contents | FreqBand | MaxFre | MinFre |
| Default | 0x02 | 0x31 | 0x00 |

FreqBand：RF band

0：reserve.

1：Chinese band2。		Fs = 920.125 + N * 0.25 (MHz) N∈[0, 19]。

2：US band				Fs = 902.75 + N * 0.5 (MHz) N∈[0,49]。

3：Korean band			Fs = 917.1 + N * 0.2 (MHz) N∈[0, 31]。

4：EU band				Fs = 865.1 + N*0.2(MHz) N∈[0, 14]。

MaxFre：the Maximum frequency controller frequency points. The scope for the N

MinFre：the Minimum frequency controller frequency points. MinFre mast smaller than MaxFre.

### 4.2.19 CFG35：Power

This configuration is defined the power on antenna

| Byte | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contents | ANT1P | ANT2P | ANT3P | ANT4P | ANT5P | ANT6P | ANT7P | ANT8P |
| Default | 0x1E | 0x1E | 0x1E | 0x1E | 0x1E | 0x1E | 0x1E | 0x1E |

ANTnP：Antenna n Power output，range is 0-30, 30≈1W.

### 4.2.20 CFG36：Antenna

This configuration is defined the effective antenna

| Byte | 0 |
| --- | --- |
| Contents | ANT |
| Default | 0x0F |

ANT：effective antenna

Bit0:Channel 1 antenna 1，Bit1 Channel 1 antenna 2

Bit2:Channel 1 antenna 3，Bit3:Channel 1 antenna 4

Bit4:Channel 2 antenna 1，Bit5:Channel 2 antenna 2

Bit6:Channel 2 antenna 3，Bit7:Channel 2 antenna 4

### 4.2.21 CFG50：IP Address

This configuration is defined the network module IP address

| Byte | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contents | IP-Address |  |  |  | SubnetMask |  |  |  |
| Default | 0xC0 | 0xA8 | 0x01 | 0xC0 | 0xFF | 0xFF | 0xFF | 0xFF |
| Byte | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| Contents | GW-Address |  |  |  | DHCP | 0x00 | 0x00 | 0x00 |
| Default | 0xC0 | 0xA8 | 0x01 | 0x01 | 0x00 | 0x00 | 0x00 | 0x00 |

IP-Address：local IP adderss.

SubnetMask：Subnet mask.

GW-Address：gate way IP address.

DHCP：DHCP switch,0x00-disable，0x01-enable

### 4.2.22 CFG51：Connection

This configuration is defined the network remote server IP and port.

| Byte | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contents | SERVERIP-Address |  |  |  | RemotePort |  | LocalPort |  | ConnectionMode |
| Default | 0xC0 | 0xA8 | 0x01 | 0xC0 | 0x2E | 0xE0 | 0x17 | 0x70 | 0x00 |

SERVERIP-Address：remote server IP address when work as TCP client

RemotePort：remote server port when work as TCP client

LocalPort：local port when work as TCP server

ConnectionMode：0x00 –TCP server，0x01-TCP client

### 4.2.23 CFG52：Repeat Connect

This configuration is defined the network repeat connect paramete

| Byte | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Contents | TCPTimeOut |  | TCPReconnectMode | 0x00 |
| Default | 0x02 | 0x58 | 0x00 | 0x00 |

TCPTimeOut：Controller as server, the continuous not to receive the client's data is still keep the connection time. High byte in the front. Unit: second. 0 means keep TCPReconnectMode：The controller as the client, an active way of connecting to a remote server. 0 = connection immediately, 1 = have a data connection

### 4.2.24 CFG53：HeartBeat and keepalive

This configuration is defined the network heartbeat and keep alive time.

| Byte | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contents | HeartbeatEn | 0x00 | HeartbeatTime |  | keepaliveEn | 0x00 | keepaliveTime |  |
| Default | 0x00 | 0x00 | 0x00 | 0x3C | 0x01 | 0x00 | 0x00 | 0x0A |

HeartbeatEn：heartbeat enable flag,0x00-disable，0x01-enable

HeartbeatTime：Heartbeat packet interval, the network free time is more than this time, will take the initiative to send a heartbeat packets. The heartbeat packet format defined in the table below.

keepaliveEn：keep alive enable flag,0x00- disable，0x01- enable

keepaliveTime：Keepalive time interval, the network free time is more than this time, will take the initiative to send keepalive packet

Heartbeat packet upload format：

| Len | Addr | Cmd | Status | Data[] |  | CRC-16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Time | PacketNo |  |  |
| 0x0F | 0xXX | 0xEF | 0x00 | 6bytes | 4bytes | LSB | MSB |

Time: Time：The message of time. From left to right, each byte followed by days, hours, minutes, seconds, milliseconds (high byte), ms (low byte). USES the system 24 hours a day. Milliseconds for two bytes, the range of 0 ~ 999, the high byte in the front.

PacketNo: Heartbeat package serial number. the high byte in the front. Controller sent up every heartbeat packets, a heartbeat packet number automatically add 1. If the heartbeat package serial number beyond 4 bytes, the serial number will back to 0 and start there

### 4.2.25 CFG14：Cascade door set

Set Controller work mode as cascade door

| Byte | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Contents | SyncMode | 0x00 | 0x00 | 0x00 |
| Default | 0x01 | 0x00 | 0x00 | 0x00 |

SyncMode：0x00-host，0x01-client

# 5.Append

| Value | describe |
| --- | --- |
| 0x00 | Success |
| 0x01 | Command length error |
| 0x02 | Command Not support |
| 0x03 | Parameter range error |
| 0x04 | Command can not doing at this moment |
| 0x06 | FLASH operate error |
| 0x07 | Command operate error |
| 0x30 | Communication error |
| 0x31 | CRC check error |
| 0x37 | Invalid handle |
| 0xEE | Command code error |